#### To Determine City Workplace Compliance with Generally Accepted Safety Standards

| Facility or Building Inspected |  | Date of Inspection |
| --- | --- | --- |
| Department |  | Department Head |

## Walking And Working Surfaces

|  | ##### Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Are all passageways, storerooms, and service rooms clean and orderly and in a sanitary condition? | §1910.22(a)(1) |  |
| Yes No | Are all workroom floors clean and as far as possible, dry? | §1910.22(a)(2) |  |
| Yes No | Are all floors, working places and passageways free from protruding nails splinters, holes or loose boards? | §1910.22(a)(3) |  |
| Yes No | Are aisles and passageways kept clear and in good repair, with no obstructions that could create a hazard? | §1910.22(b)(1) |  |
| Yes No | Are permanent aisles and passageways appropriately marked? | §1910.22(b)(2) |  |
| Yes No | Are covers and guardrails provided to protect from hazards of open pits, tanks, vats, ditches, etc. | §1910.22(c) |  |
| Yes No | Are floor loading protection signs supplied and securely affixed? | §1910.22(d)(1) |  |
| Yes No | Are there any loads on any floor or roof greater than the approved weight? | §1910.22(d)(2) |  |
| Yes No | Are standard railings provided at stairway floor openings? | §1910.23(a)(1) |  |
| Yes No | Are standard railings with toeboards provided at ladderway floor openings? | §1910.23(a)(2) |  |
| Yes No | Are hatchway and chute openings guarded by either a hinged floor cover or railings? | §1910.23(a)(3) |  |
| Yes No | Are trapdoors and manholes protected by a guard or railings when opened? | §1910.23(a)(5) & (6) |  |
| Yes No | Are all floor holes that a person cannot accidentally walk because of equipment, covered? | §1910.23(a)(9) |  |
| Yes No | Are open-sided floors, platforms & runways, four feet or more above adjacent level, guarded by a standard railing? | §1910.23(c)(1) |  |
| Yes No | Are toe-boards included, if beneath the open sides a person can pass, there is moving machinery or other equipment which falling materials could cause a hazard? | §1910.23(c)(1)(I) – (iii) |  |
| Yes No | Are all runways, four feet or more above the adjacent level equipped with standard railings? | §1910.23(c)(2) |  |
| Yes No | Are open-sided floors near hazards guarded with a standard railing and toeboard? | §1910.23(c)(3) |  |
| Yes No | Are stairways <44 inches wide, with no open sides, equipped with at least one handrail, preferably R/side descending? | §1910.23(d)(1)(i) |  |
| Yes No | Are stairways <44 inches wide, with one open side, equipped with at least one stair railing on the open side? | §1910.23(d)(1)(ii) |  |
| Yes No | Are stairways <44 inches wide, with both sides open, equipped with stair railing on both sides? | §1910.23(d)(1)(iii) |  |
| Yes No | Are fixed stairs installed where required – Daily or regular usage or carrying tools or equipment by hand? | §1910.24(b) |  |
| Yes No | Do fixed stairways have a minimum stair width of 22”? | §1910.24(d) |  |
| Yes No | Are railings and handrails provided as required? | §1910.24(h) |  |
| Yes No | Are all portable wood ladders free from sharp edges and splinters? | §1910.25(b)(1)(i) |  |
| Yes No | Are portable wood and metal ladders maintained in good condition at all times? | §1910.25(d)(1)(i) §1910.26(c)(2)(iv) |  |
| Yes No | Are portable wood ladders inspected “frequently” and those with defects removed from service? | §1910.25(d)(1)(x) |  |
| Yes No | Does the facility have any fixed ladders? See §1910.27 for specific requirements for fixed ladders. |  |  |
| Yes No | Does the facility have any scaffolding in use? See §1910.28 for specific requirements for scaffolding. |  |  |
| Yes No | Are dock-boards or bridge plates secured in position by anchoring or be devices to prevent their slipping? | §1910.30(a)(2) |  |
| Yes No | Do portable dock-boards have handholds? | §1910.30(a)(4) |  |

## Means of Egress

|  | Requirement | ##### Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Are exits provided sufficient to permit the prompt escape of occupants in case of fire or other emergency? | §1910.36(b)(1) |  |
| Yes No | Are building exits arranged so as to provide unobstructed egress (with no inside locks)? | §1910.36(b)(4) |  |
| Yes No | Are exits or the route to reach them conspicuously indicated” | §1910.36(b)(5) |  |
| Yes No | Is adequate and reliable illumination provided for all exit facilities? | §1910.36(b)(6) |  |
| Yes No | If necessary does the facility have a fire alarm system? | §1910.36(b)(7) |  |
| Yes No | Are at least two means of egress provided for each area of the building? | §1910.36(b)(8) |  |
| Yes No | During repairs, alterations or remodeling, are existing exits continuously maintained, or in lieu thereof, other measures taken which provide equivalent safety? | §1910.36(c)(2) |  |
| Yes No | Are all protection systems,(sprinkler, fire detection & alarm where installed) in proper operating condition? | §1910.36(d)(2) |  |
| Yes No | Are exits readily accessible at all times? | §1910.37(f)(1) |  |
| Yes No | Are exit doors and the ways of exit thereto, clearly recognizable as such? | §1910.37(f)(4) |  |
| Yes No | Are paths of exit access at least 28 inches wide? | §1910.37(f)(6) |  |
| Yes No | Do all exits discharge directly to the street, or to a yard, court or other open space? | §1910.37(h)(1) |  |
| Yes No | Are the means of egress free of obstructions? | §1910.37(k)(2) |  |
| Yes No | Are automatic sprinkler systems continuously maintained and inspected and tested periodically? | §1910.37(m) |  |
| Yes No | Are fire alarm systems maintained and tested? | §1910.37(n) |  |
| Yes No | Are all exits marked? | §1910.37(q)(1) |  |
| Yes No | Are non-exits that may be mistaken for exits marked "Not an Exit", or by a sign indicating its actual use? | §1910.37(q)(2) |  |
| Yes No | Are all exit signs readily visible and of distinctive color? | §1910.37(q)(3)&(4) |  |
| Yes No | Are exit signs with arrows placed where direction of exit is not obvious? | §1910.37(q)(5) |  |
| Yes No | Are exit signs illuminated? | §1910.37(q)(6) |  |

## Emergency Action Plans

| Yes No | Does the facility have an Emergency Action Plan, (EAP), for all foreseeable emergencies? | §1910.38(a)(1) |  |
| --- | --- | --- | --- |
| Yes No | Does the EAP have the following elements: | §1910.38(a)(2) |  |
| Yes No | Emergency escape procedures and route assignments? | §1910.38(a)(2)(i) |  |
| Yes No | Procedures to be followed by employees who remain to operate critical plant operations before they evacuate” | §1910.38(a)(2)(ii) |  |
| Yes No | A procedure to account for all employees after the evacuation is completed? | §1910.38(a)(2)(iii) |  |
| Yes No | Rescue and medical duties for those employees who are to perform them? | §1910.38(a)(2)(iv) |  |
| Yes No | The preferred means of reporting fires and other emergencies? | §1910.38(a)(2)(v) |  |
| Yes No | The names or job titles of persons who can be contacted for further information about the plan? | §1910.38(a)(2)(vi) |  |
| Yes No | Does the facility have an employee alarm system? | §1910.38(a)(3)(i) |  |
| Yes No | Are a sufficient number of personnel trained to assist in the safe and orderly evacuation of employees? | §1910.38(a)(5)(i) |  |
| Yes No | Is the EAP reviewed with each covered employee initially and/or when the plan is changed? | §1910.38(a)(5)(ii) |  |
| Yes No | Is the written plan available for review by employees? (Employers with 10 or more employees may communicate the details orally.) | §1910.38(a)(5)(iii) |  |
| Yes No | Does the Fire Protection Plan, (FPP), list the major workplace fire hazards? | §1910.38(b)(2)(i) |  |
| Yes No | Does the FPP list the names or job titles of those persons responsible for maintenance of equipment and systems to prevent or control fires? | §1910.38(b)(2)(ii) |  |
| Yes No | Does the FPP list the names or job titles of those persons responsible for control of fuel source hazards? | §1910.38(b)(2)(iii) |  |
| Yes No | Are housekeeping procedures included in the FPP to prevent the accumulation of flammable and combustible waste materials? | §1910.38(b)(3) |  |
| Yes No | Are employees trained about the fire hazards of the materials to which they are exposed? | §1910.38(b)(4)(i) |  |

## Powered Platforms, Manlifts, etc.

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Are the lift controls for “extensible boom platforms” tested daily to determine they are in safe working condition? | §1910.67(c)(2)(ii) |  |
| Yes No | Are only properly trained personnel allowed to operate aerial lift vehicles? | §1910.67(c)(2)(ii) |  |
| Yes No | Are body belts with lanyard attached to the boom or basket worn when working from aerial lifts? | §1910.67(c)(2)(v) |  |

## Occupational Health & Environmental Controls

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Are any sandblasting operations done? See §1910.94 for specific requirement for ventilation in sandblasting oper. |  |  |
| Yes No | Are controls being utilized to prevent exposure to excessive noise? | §1910.95(b)(1) |  |
| Yes No | Does the Company have a Hearing Conservation Program? | §1910.95(c)(1) |  |
| Yes No | Are employees in the HCP being monitored for noise exposure? | §1910.95(d)(1) |  |
| Yes No | Is there an audiometric testing program? | §1910.95(g)(1) |  |
| Yes No | Are certain employees tested with annual audiograms? | §1910.95(g)(6) |  |
| Yes No | Are certain employees wearing hearing protection as required? | §1910.95(i)(2)(i) |  |
| Yes No | Are employees in the HCP receiving proper training? | §1910.95(k)(1) |  |
| Yes No | Have copies of the standard been made available to employees? | §1910.95(l)(1) |  |
| Yes No | Does the facility have any Ionizing Radiation operations? See §1910.96 for specific requirements concerning Ionizing Radiation. |  |  |

## Hazardous Materials

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Are compressed gas cylinders visually inspected to determine they are in a safe condition? | §1910.101(a) |  |
| Yes No | Are compressed gases handled in accordance with the Compressed Gas Assoc. Pamphlet P-1-1965? | §1910.101(b) |  |
| Yes No | Are inside storage rooms constructed according to the fire-resistive rating for their use? | §1910.106(d)(4)(i) |  |
| Yes No | Are flammable liquids kept in covered containers when not in use? | §1910.106(e)(2) (iv)(a) |  |
| Yes No | Are portable fire extinguishers provided in the areas of operation and storage? | §1910.106(e)(5)(i) |  |
| Yes No | Are all fire protection facilities maintained and tested? | §1910.106(e)(5)(v) |  |
| Yes No | Have adequate precautions been taken to prevent the ignition of flammable vapors? | §1910.106(e)(6)(i) |  |
| Yes No | Are combustible waste materials and residue kept to a minimum, stored in covered metal containers and disposed of daily? | §1910.106(e)(9)(iii) |  |
| Yes No | Are the grounds around buildings kept free of weeds, trash or other unnecessary combustible materials? | §1910.106(e)(9)(iv) |  |
| Yes No | Does the facility have any spray booth operations? See §1910.107 for specifics concerning spray booth operations. |  |  |
| Yes No | Does the facility have any dip tank operations? See §1910.108 for specifics concerning dip tank operations. |  |  |
| Yes No | Does the facility have any blasting operations? See §1910.109 for specifics concerning explosives and blasting agent operations. |  |  |
| Yes No | Are LP and other gas containers properly marked? | §1910.110(b)(5)(iii) |  |
| Yes No | Is there a current Emergency Response Plan for hazardous substance release? | §1910.120(q)(1) |  |

## Personal Protective Equipment

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Is Personal Protective Equipment (PPE) provided, used, and maintained in a sanitary and reliable condition wherever it is necessary? | §1910.132(a) |  |
| Yes No | Has a hazard assessment of the workplace been completed? | §1910.132(d)(1) |  |
| Yes No | Has the proper PPE been selected? | §1910.132(d)(1)(i) |  |
| Yes No | Have the selection decisions been communicated to the employees? | §1910.132(d)(1)(ii) |  |
| Yes No | Has the PPE selected been properly fitted to the employees? | §1910.132(d)(1)(iii) |  |
| Yes No | Has a written certification of hazard assessment been completed? | §1910.132(d)(2) |  |
| Yes No | Are employees properly trained in the use of PPE? | §1910.132(f)(1) |  |
| Yes No | Has the employer verified the training through a written certification? | §1910.132(f)(4) |  |
| Yes No | Are employees required to use face protection when necessary? | §1910.133(a)(1) |  |
| Yes No | Are respirators provided by the employer when necessary? | §1910.134(a)(2) |  |
| Yes No | Are there written standard operating procedures governing the selection and use of respirators? | §1910.134(b)(1) |  |
| Yes No | Are respirator users properly instructed and trained? | §1910.134(b)(3) |  |
| Yes No | Are respirators stored in convenient, clean and sanitary conditions? | §1910.134(b)(6) |  |
| Yes No | Are respirators worn routinely inspected during cleaning and those for emergency use inspected monthly? | §1910.134(b)(7) |  |
| Yes No | Is a record of the inspection dates and findings kept for emergency respirators? | §1910.134(f)(2)(iv) |  |
| Yes No | Is the program regularly inspected and evaluated to determine effectiveness? | §1910.134(b)(9) |  |
| Yes No | Are all persons assigned to tasks requiring the use of respirators, physically able to perform the work and use the equipment? | §1910.134(b)(10) |  |
| Yes No | Has a physician determined what health and physical conditions are pertinent in evaluating whether employees are able to wear respirators? | §1910.134(b)(10) |  |
| Yes No | Is the medical status of respirator users reviewed periodically (annually)? | §1910.134(b)(10) |  |
| Yes No | Are respirator users properly instructed in selection, use and maintenance of their respirators? | §1910.134(e)(5) |  |
| Yes No | Do respirator users receive fitting instructions including demonstrations and practice? | §1910.134(e)(5)(i) |  |
| Yes No | Are employees required to wear protective helmets when working in areas where there is a potential for injury from falling objects? | §1910.135(a)(1) |  |
| Yes No | Are employees required to wear protective footwear when working in areas where there is a danger of foot injuries due to falling or rolling objects, objects piercing the sole, or exposure to electrical hazards? | §1910.136(a) |  |
| Yes No | Are employees required to wear appropriate hand protection when hands are exposed to hazards? | §1910.138(a) |  |

## General Environmental Controls

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Is the facility kept clean to the extent that the nature of the work allows? | §1910.141(a)(3)(i) |  |
| Yes No | Are the floors of workrooms maintained in a dry condition? | §1910.141(a)(3)(ii) |  |
| Yes No | Where wet processes are used, is drainage maintained and dry standing places provided? | §1910.141(a)(3)(ii) |  |
| Yes No | Are waste receptacles maintained in a sanitary condition? | §1910.141(a)(4)(i) |  |
| Yes No | Is refuse and garbage removed in a manner to avoid creating a health menace? | §1910.141(a)(4)(ii) |  |
| Yes No | Are portable drinking water dispensers maintained in sanitary conditions? | §1910.141(b)(1)(iii) |  |
| Yes No | Do lavatories have hot & cold water and hand soap? | §1910.141(d)(2)(ii) & (iii) |  |
| Yes No | Has the workplace been evaluated for permit-required confined spaces (PRCS)? | §1910.146(c)(1) |  |
| Yes No | Are “danger signs” posted by the PRCS? | §1910.146(c)(2) |  |
| Yes No | If employees are to enter PRCS, is there a written permit space program? | §1910.146(c)(4) |  |
| Yes No | If non-employees are to enter PRCS, do you, the host employer, allow entrance only in compliance the standard? | §1910.146(c)(8)(i) |  |
| Yes No | Are affected employees given permit-required confined space training? | §1910.146(g)(1) |  |
| Yes No | Does the facility have an energy control program (Lockout/Tagout)? | §1910.147(c)(1) |  |
| Yes No | Have energy control procedures been developed and documented? | §1910.147(c)(4)(i) |  |
| Yes No | Do the procedures clearly and specifically outline the scope, purpose, authorization, rules, and techniques to be utilized for the control of hazardous energy? | §1910.147(c)(4)(ii) |  |
| Yes No | Are all protective materials and hardware provided by the employer? | §1910.147(c)(5)(i) |  |
| Yes No | Are the lockout/tagout devices standardized throughout the facility? | §1910.147(c)(5)(ii) (B) |  |
| Yes No | Is at least an annual inspection of energy control procedures conducted? | §1910.147(c)(6)(i) |  |
| Yes No | Is an inspection conducted of each authorized employee, reviewing the employee’s responsibilities under the energy control procedure being inspected? | §1910.147(c)(6)(i) (C) |  |
| Yes No | Are the inspections certified by the employer? | §1910.147(c)(6)(ii) |  |
| Yes No | Are all employees trained in the energy control program? | §1910.147(c)(7)(i) |  |
| Yes No | Has the employer certified that the training has been accomplished and is being kept up to date? | §1910.147(c)(7)(iv) |  |
| Yes No | Are lockout/tagout devices properly affixed by authorized employees? | §1910.147(d)(4)(i) |  |
| Yes No | Whenever outside workers are engaged in activities covered by standard, are LOTO procedures exchanged? | §1910.147(f)(2)(i) |  |

## MEDICAL SERVICES AND FIRST AID

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Do you require each employee to have a physical examination? | §1910.120(q)(9)(i) §1910.1001(l)(2)(ii) | Required for certain occupations such as HAZMAT, Asbestos, etc. |
| Yes No | Are medical personnel readily available for advice and consultation on matters of plant health? | §1910.151(a) |  |
| Yes No | Is there a hospital, clinic, or infirmary for medical care in proximity of your workplace? |  |  |
| Yes No | If medical and first aid facilities are not in proximity of your workplace, is at least one employee on each shift currently qualified to rend first aid? | §1910.151(b) |  |
| Yes No | Are first aid kits easily accessible to each work area, with necessary supplies available, periodically inspected and replenished as needed? | §1910.151(b) |  |
| Yes No | Have first aid kits been approved by a physician, indicating that they are adequate for a particular area of operation? | §1910.151(b) |  |
| Yes No | Are eyewash and body washing facilities provided where employees may be exposed to corrosive materials? | §1910.151(c) |  |

## Fire Protection

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Have fire brigades been established in the facility? See §1910.156 for specific information on Fire Brigades. |  |  |
| Yes No | Are portable fire extinguishers provided for use by employees? | §1910.157(c)(1) |  |
| Yes No | Are portable fire extinguishers maintained in a fully charged and operable condition and kept in their designated place? | §1910.157(c)(4) |  |
| Yes No | Are Class A portable fire extinguishers distributed so that the distance to any extinguisher is 75 feet or less? | §1910.157(d)(2) |  |
| Yes No | Are Class B portable fire extinguishers distributed so that the distance to any extinguisher is 50 feet or less? | §1910.157(d)(4) |  |
| Yes No | Are portable fire extinguishers visually inspected monthly? | §1910.157(e)(2) |  |
| Yes No | Are portable fire extinguishers subjected to an annual maintenance check? | §1910.157(e)(3) |  |
| Yes No | Are portable fire extinguishers subjected to hydrostatically tested as required? | §1910.157(f)(2) |  |
| Yes No | Are employees trained in the general principles of fire extinguisher use? | §1910.157(g)(1) |  |
| Yes No | Are employees trained initially and annually in portable fire extinguisher use? | §1910.157(g)(2) |  |

## Compressed Gas and Compressed Air Equipment

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Are air receivers equipped with pressure gages and valves? | §1910.169(b)(3)(i) |  |
| Yes No | Are safety valves be tested frequently and at regular intervals? | §1910.169(b)(3)(iv) |  |

## Material Handling and Storage

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Is sufficient safe clearance allowed where mechanical handling equipment is used? | §1910.176(a) |  |
| Yes No | Is material stacked and stored such that a hazard is not created? | §1910.176(b) |  |
| Yes No | Are storage areas kept free from an accumulation of materials that constitute hazards? | §1910.176(c) |  |
| Yes No | Are clearance signs posted? | §1910.176(e) |  |
| Yes No | Are covers or guardrails provided to protect workers from open pits, tanks, vats, ditches, etc.? | §1910.176(g) |  |
| Yes No | Does the manufacturer approve any modifications to powered industrial trucks? | §1910.178(a)(4) |  |
| Yes No | Are all nameplates and markings maintained in a legible condition? | §1910.178(a)(6) |  |
| Yes No | Are only properly designated trucks used in approved areas? | §1910.178(c) |  |
| Yes No | Are all lift trucks equipped with an overhead guard? | §1910.178(e)(1) |  |
| Yes No | Do fuel handling and storage facilities meet the requirements of NFPA? | §1910.178(f) |  |
| Yes No | Are battery charging installations located in areas designated for that purpose? | §1910.178(g)(1) |  |
| Yes No | Are facilities provided for flushing spilled electrolyte, for fire protection and for protecting charging equipment for damage? | §1910.178(g)(2) |  |
| Yes No | Is a hoist or similar equipment provided for handling batteries? | §1910.178(g)(4) |  |
| Yes No | Is smoking prohibited in the charging area? | §1910.178(g)(10) |  |
| Yes No | Have precautions been taken to prevent open flames, sparks or electric arcs in the charging area? | §1910.178(g)(11) |  |
| Yes No | Do the trucks have lights? | §1910.178(h)(2) |  |
| Yes No | Have steps been take to prevent the build-up of carbon monoxide gas? | §1910.178(I)(1) |  |
| Yes No | Are trailer wheels chocked when boarded by powered industrial trucks? | §1910.178(k)(1) §1910.178(m)(7) |  |
| Yes No | Are only properly trained personnel allowed to operate powered industrial trucks? See Training Requirements for details on LTO training. | §1910.178(l) |  |
| Yes No | When left unattended, is the lift lowered, controls neutralized, power shut off and brakes applied: | §1910.178(m)(5)(i) |  |
| Yes No | Is passing of trucks traveling in the same direction prohibited? | §1910.178(n)(3) |  |
| Yes No | Are drivers required to slow down and sound the horn when crossing aisles and at other locations where vision is obstructed? | §1910.178(n)(4) |  |
| Yes No | Are unsafe powered industrial trucks taken out of service until repaired? | §1910.178(p)(1) |  |
| Yes No | Is proper care taken when filling fuel tanks including stopping engine? | §1910.178(p)(2) |  |
| Yes No | Are industrial trucks examined before being put into service each day? Are records kept? | §1910.178(q)(7) |  |
| Yes No | On cranes and hoists, are functional operating mechanisms inspected daily? | §1910.179(j)(2)(i) |  |
| Yes No | Are crane hooks with deformation or cracks inspected daily? | §1910.179(j)(2)(iii) |  |
| Yes No | Are cranes inspected periodically? | §1910.179(j)(3) |  |
| Yes No | Has a preventive maintenance program been established? | §1910.179(l)(1) |  |
| Yes No | Are ropes inspected at least monthly with proper documentation? | §1910.179(m)(1) |  |

## Machinery and Machine Guarding

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Are machine guarding methods provided to protect operators for danger? | §1910.212(a)(1) |  |
| Yes No | Are guards provided at points of operation? | §1910.212(a)(3)(ii) |  |
| Yes No | Are guards provided for fan blades less than 7’ above the floor or working level? | §1910.212(a)(5) |  |
| Yes No | Is machinery designed for fixed locations, securely anchored to prevent walking or moving? | §1910.212(b) |  |
| Yes No | Are provisions made to prevent automatic re-start upon restoration of power? (After a power failure) | §1910.213(b)(3) |  |
| Yes No | Are hand-fed ripsaws guarded? | §1910.213(c)(1) |  |
| Yes No | Are hand-fed ripsaws equipped with spreaders? | §1910.213(c)(2) |  |
| Yes No | Are hand-fed ripsaws equipped with non-kickback fingers or dogs? | §1910.213(c)(3) |  |
| Yes No | Are circular crosscut table saw guarded by a hood? | §1910.213(d)(1) |  |
| Yes No | Are radial saws equipped with upper hood guards? | §1910.213(h)(1) |  |
| Yes No | Are radial saws installed with the front edge slightly higher than the back? | §1910.213(h)(4) |  |
| Yes No | Are band saws and band resaws properly guarded? | §1910.213(i)(1) |  |
| Yes No | Are abrasive wheels used only on machines that are properly guarded? | §1910.215(a)(1) |  |
| Yes No | Does the guard on abrasive wheel machinery cover the spindle end, nut and flange? | §1910.215(a)(2) |  |
| Yes No | On abrasive wheel machinery are work rests provided? | §1910.215(a)(4) |  |
| Yes No | Are foot pedal mechanisms protected to prevent unintended operation? | §1910.217(b)(4)(i) |  |
| Yes No | Are two-handed controls protected to prevent unintended operation, and arranged to require the use of both hands? | §1910.217(b)(6)(i) |  |
| Yes No | Are red stop controls provided? | §1910.217(b)(6)(ii) |  |
| Yes No | Are point of operation guards provided on all mechanical power presses? | §1910.217(c)(1)(i) |  |
| Yes No | Does the guard prevent to entry of fingers or hands into the point of operation? | §1910.217(c)(2)(i) (a) |  |
| Yes No | Are mechanical power presses subjected to periodic and regular inspections, and are certification records maintained? | §1910.217(e)(1)(i) |  |
| Yes No | Are mechanical power presses inspected and tested at least weekly? Are records of inspections maintained? | §1910.217(e)(1)(ii) |  |
| Yes No | Are personnel caring for, inspecting and maintaining mechanical power presses, properly trained? | §1910.217(e)(3) |  |
| Yes No | Are operators of mechanical power presses properly trained? | §1910.217(f)(2) |  |
| Yes No | Are flywheels located 7’ or less from the floor guarded? | §1910.219(b)(1) |  |
| Yes No | Are all exposed parts of horizontal shafting located 7’ or less from the floor guarded? | §1910.219(c)(2)(i) |  |
| Yes No | Are vertical and incline shafting located 7’ or less from the floor guarded? | §1910.219(c)(3) |  |
| Yes No | Are projecting shaft ends guarded? | §1910.219(c)(4)(i) |  |
| Yes No | Are pulleys located 7’ or less from the floor guarded? | §1910.219(d)(1) |  |
| Yes No | Are horizontal belts and ropes properly guarded? | §1910.219(e)(1)(i) |  |
| Yes No | Are vertical and inclined belts properly guarded? | §1910.219(e)(3)(i) |  |
| Yes No | Are gears properly guarded? | §1910.219(f)(1) |  |
| Yes No | Are sprockets and chains properly guarded? | §1910.219(f)(3) |  |
| Yes No | Are couplings smooth with no screws or bolts projecting beyond the collar? | §1910.219(i)(2) |  |

## Hand & Portable Powered Tools

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Is the employer responsible for safe condition of hand tools, including those supplied by the employees? | §1910.242(a) |  |
| Yes No | Is compressed air used for cleaning only when reduced to 30 p.s.i.? | §1910.242(b) |  |
| Yes No | Are portable circular saws equipped with guards? | §1910.243(a)(1)(i) |  |
| Yes No | Are all portable electric powered tools properly grounded in accordance with subpart S? | §1910.243(a)(5) |  |
| Yes No | Are abrasive wheels on portable tools properly guarded? | §1910.243(c)(1) |  |

## Welding, Cutting and Brazing

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Are proper steps taken to prevent fires when welding? | §1910.252(a)(1) |  |
| Yes No | Is proper PPE used when welding? | §1910.252(b)(1) |  |
| Yes No | Are welding screens used to protect other persons? | §1910.252(b)(2)(iii) |  |
| Yes No | Are compressed gas cylinders properly marked, identifying the gas content? | §1910.253(b)(1)(ii) |  |
| Yes No | Are cylinders over 30lbs. equipped with valve protection caps? | §1910.253(b)(2)(iv) |  |
| Yes No | When stored Inside buildings, are cylinders stored in a suitable location? | §1910.253(b)(2)(ii) |  |
| Yes No | Are oxygen cylinders separated from fuel-gas cylinders when stored? | §1910.253(b)(4)(iii) |  |
| Yes No | Are oxygen cylinders stored away from highly combustible materials? | §1910.253(b)(4)(i) |  |
| Yes No | Are workers assigned to operate arc welding equipment “acquainted” with the provisions of §1910.252 a, b & c? | §1910.254(d)(1) |  |
| Yes No | Are cables with splices within 10’ of the holder, prohibited form being used? | §1910.254(d)(8) |  |
| Yes No | Are cables with damaged insulation replaced before use? | §1910.254(d)(9)(iii) |  |

## Electrical

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | Has electrical equipment been examined to be sure it is free of recognized hazards? | §1910.303(b)(1) |  |
| Yes No | Is electric equipment installed according to instructions? | §1910.303(b)(2) |  |
| Yes No | Are disconnecting means properly marked, including the circuits in panel boxes? | §1910.303(f) |  |
| Yes No | Are live parts properly guarded? | §1910.303(g)(2)(i) |  |
| Yes No | Are all metal-enclosed switchgear, pull boxes, connection boxes and similar associated equipment marked with “Caution” signs? | §1910.303(h)(2)(ii) |  |
| Yes No | Is sufficient workspace provided around electrical equipment? | §1910.303(h)(3)(i) |  |
| Yes No | Are warning signs posted around high voltage? | §1910.304(d)(2)(ii) |  |
| Yes No | If circuit breakers are used as switches for 120-volt fluorescent lighting, are they approved for such purpose and marked “SWD”? | §1910.304(e)(1)(vi)(C) |  |
| Yes No | Is equipment connected by cord and plug properly grounded? | §1910.304(f)(5)(v) |  |
| Yes No | Is the use of temporary wiring in permanent installations prohibited? | §1910.305(a)(2) |  |
| Yes No | Are covers and canopies on all boxes? | §1910.305(b)(2) |  |
| Yes No | Is the use of flexible cords approved and suitable? | §1910.305(g)(1)(i) |  |
| Yes No | Is the improper use of flexible cords prohibited? (Substitute for fixed wiring, through walls, doors ceilings, etc.) | §1910.305(g)(1)(iii) |  |
| Yes No | Are flexible cords used only in continuous lengths? | §1910.305(g)(2)(ii) |  |
| Yes No | Is strain relief for flexible cords provided? | §1910.305(g)(2)(iii) |  |
| Yes No | Are employees trained in electrical safety related work practices? | §1910.332(b)(1) |  |
| Yes No | Is all portable cord and plug equipment visually inspected before use? | §1910.334(a)(2)(i) |  |
| Yes No | Is any damaged equipment removed from service? | §1910.334(a)(2)(ii) |  |
| Yes No | Are plugs inspected to make sure they are not altered? | §1910.334(a)(3)(ii) |  |
| Yes No | Is proper PPE provided as required? | §1910.335(a)(1)(i) |  |
| Yes No | Are safety signs, tags or symbols used where necessary to warn employees about electrical hazards? | §1910.335(b)(1) |  |

## Toxic and Hazardous Substances

|  | Requirement | Standard | Comments |
| --- | --- | --- | --- |
| Yes No | 8-hour time weighted averages | §1910.1000(a)(2) |  |
| Yes No | 8-hour time weighted average exposure limits | §1910.1000(c) |  |
| Yes No | Keeping employee exposure to air contaminants within prescribed limits | §1910.1000(e) |  |
| Yes No | Has a building inspection been completed to determine the presence of asbestos containing materials? | §1910.1001(j)(2)(i) |  |
| Yes No | Are records of inspections including the location of ACM and PACM maintained? | §1910.1001(j)(2)(ii) |  |
| Yes No | If ACM or PACM are present, have employees been notified? Note: If ACM is present, many other rules are triggered. | §1910.1001(j)(2)(iii) |  |
| Yes No | Are signs posted warning employees that asbestos is present? | §1910.1001(j)(3)(i) |  |
| Yes No | Does this facility contain lead? | §1910.1025(c)(1) |  |
| Yes No | Have tests been conducted to determine the exposure levels? | §1910.1025(d)(2) |  |
| Yes No | Have affected employees been informed of possible lead exposure? | §1910.1025(l)(1)(i) |  |
| Yes No | Does the company have an Exposure Control Plan for bloodborne pathogens? | §1910.1030(c)(1)(i) |  |
| Yes No | Has an exposure determination been completed? | §1910.1030(c)(2)(i) |  |
| Yes No | Are Universal precautions observed? | §1910.1030(d)(1) |  |
| Yes No | Is the Hepatitis B vaccination available to all employees who have an occupational exposure to BBP? | §1910.1030(f)(2)(i) |  |
| Yes No | Is proper communication of hazards to employees (information and training) conducted? | §1910.1030(g)(2)(i) |  |
| Yes No | Does the company have a written hazard communication (HazCom) program? | §1910.1200(e)(1) |  |
| Yes No | Does the program contain a list of all hazardous chemicals present? | §1910.1200(e)(1)(i) |  |
| Yes No | Is the HazCom program available to employees? | §1910.1200(e)(4) |  |
| Yes No | Is each container of hazardous chemicals labeled? | §1910.1200(f)(5) |  |
| Yes No | Are there Material safety data sheets for every chemical in the workplace? | §1910.1200(g)(1) |  |
| Yes No | Are MSDS’s readily accessible to employees in their work areas? | §1910.1200(g)(8) |  |
| Yes No | Has information and training on hazardous chemicals been provided to employees? | §1910.1200(h) |  |